Robotica (year), page no doi:10.1017/xxxxx

SUPPLEMENTARY MATERIAL

# A Scoping Review on Lower Limb Exoskeleton Actuation's 
Description and Characteristics

Francesco Bettella1, Stefano Tortora*1,2, Emanuele Menegatti1,2, Nicola Petrone3 and Alessandra Del Felice1,4

1Padova Neuroscience Center, University of Padova, Italy.

2Department of Information Engineering, University of Padova, Italy.

3Department of Industrial Engineering, University of Padova, Italy.

4Department of Neuroscience, Section of Neurology, University of Padova, Italy.

*Corresponding author. E-mail: stefano.tortora@unipd.it.

Table S1. In this table, all papers that meet the inclusion criteria of this study are listed. For each exoskeleton, the first two columns show the rating scores of completeness of description of motors and reducers. The main technical parameters of motors and reducers of the hip and knee joints, expressly declared in the papers or from external resources as catalogues, are also reported.

Legend: R: Rehabilitative; A: Assistive; P: Prototype; ND: Not Defined; NP: Not Present; BLDC: Brushless DC Motor; BDC: Brushed DC Motor; HD: Harmonic Drive; Planetary Gearbox;

|  | RATING |  |  | HIP MOTOR |  |  |  |  |  | HIP REDUCTION |  |  | KNEE MOTOR |  |  |  |  |  | KNEE REDUCTION |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ref. | Motor | Reducer | App. | Type | Voltage [V] | Max Power [W] | Torque [Nm] | Speed [rpm] | Weight [kg] | Type | Ratio | Weight [kg] | Type | Voltage [V] | Power [W] | Torque [Nm] | Speed [rpm] | Weight [kg] | Type | Ratio | Weight [kg] |
| Tian et al 2024 [62] | 1 | 1 | R | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND |
| Soliman et al 2024 [63] | 4 | 5 | A | BLDC | 48 | 280 | 0.940 | 3930 | 0.400 | HD | 100 | 0.240 | BLDC | 48 | 280 | 0.940 | 3930 | 0.400 | HD | 100 | 0.240 |
| Qiu et al 2024 [64] | 1 | 1 | P | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND |
| Li et al 2024 [65] | 1 | 1 | A | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND |
| Yu et al 2023 [66] | 5 | 4 | A | BLDC | 48 | 100 | 0.298 | 3460 | 0.355 | HD | 120 | ND | BLDC | 48 | 100 | 0.298 | 3460 | 0.355 | HD | 50 | ND |
| Plaza et al 2023 [67] | 1 | 1 | R | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND |
| Li et al 2023 [68] | 4 | NP | R | BLDC | ND | 320 | 0.560 | 3120 | 0.980 | NP | NP | NP | BLDC | ND | 320 | 0.560 | 3120 | 0.980 | NP | NP | NP |
| Gao et al 2023 [69] | 1 | 1 | P | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND |
| Fang et al 2023 [31] | 5 | 4 | A | Stepper | ND | ND | 2.400 | 210 | 0.700 | HD | 100 | ND | Stepper | ND | ND | 2.500 | 420 | 0.320 | HD | 100 | ND |
| Bergmann et al 2022 [26] | 5 | 5 | R | BLDC | 48 | 150 | 0.437 | 3230 | 0.350 | HD | 160 | ND | BLDC | 48 | 150 | 0.437 | 3230 | 0.350 | HD | 100 | ND |
| Changcheng et al 2022 [70] | 3 | 4 | R | BLDC | ND | ND | ND | ND | ND | HD | 100 | ND | BLDC | ND | ND | ND | ND | ND | PG | 46 | ND |
| Huang et al 2022 [71] | 4 | 4 | A | BLDC | ND | ND | 0.460 | 5000 | ND | HD | 160 | ND | BLDC | ND | ND | 0.460 | 5000 | ND | HD | 160 | ND |
| Nesler et al 2022 [72] | 5 | 4 | A | BLDC | 24 | ND | 1.000 | 2205 | 0.485 | PG | 9 | ND | BLDC | 24 | ND | 1.000 | 2205 | 0.485 | PG | 9 | ND |
| Pan et al 2022 [73] | 4 | 4 | R | BLDC | ND | 400 | ND | ND | ND | HD | 50 | ND | BLDC | ND | 400 | ND | ND | ND | HD | 50 | ND |
| Taza-Aquino et al 2022 [28] | 4 | 1 | R | BDC | 12 | 200 | ND | ND | ND | ND | ND | ND | BLDC | 12 | 200 | ND | ND | ND | ND | ND | ND |
| Vouga et al 2022 [74] | 4 | 4 | A | BLDC | 48 | ND | ND | ND | ND | HD | 100 | 0.650 | BLDC | ND | ND | ND | ND | ND | HD | 100 | 0.650 |
| Wang et al 2022 [29] | 3 | NP | P | BLDC | ND | ND | ND | ND | ND | NP | NP | NP | BLDC | ND | ND | ND | ND | ND | NP | NP | NP |
| Zhou et al 2022 [75] | 2 | 4 | P | BDC | 24 | 100 | ND | ND | ND | HD | 100 | ND | BDC | ND | 100 | ND | ND | ND | HD | 100 | ND |
| Hasan et al 2022 [57] | 2 | 4 | R | BLDC | ND | ND | ND | ND | ND | HD | 100:1 | ND | BLDC | ND | ND | ND | ND | ND | HD | ND | ND |
| Andrade et al 2021 [14] | 4 | 5 | R | BLDC | ND | 70 | 0.134 | 4750 | 0.150 | HD | 160 | 0.280 | BLDC | 24 | 70 | 0.134 | 4750 | 0.150 | HD | 160 | 0.280 |
| Choi et al 2021 [17] | 3 | NP | P | BLDC | 48 | 70 | 0.134 | 4750 | 0.150 | NP | NP | NP | BLDC | 24 | 70 | 0.134 | 4750 | 0.150 | NP | NP | NP |
| Christensen et al 2021 [19] | 3 | 5 | A | BLDC | ND | 100 | 0.269 | 3730 | 0.355 | HD | 125 | ND | BLDC | 24 | 100 | 0.269 | 3730 | 0.355 | HD | 60 | ND |
| Laffranchi et al 2021 [76] | 3 | 5 | A | BLDC | 36 | ND | ND | ND | ND | HD | 100 | 1.200 | BLDC | 24 | ND | ND | ND | ND | HD | 100 | 1.200 |
| Meijneke et al 2021 [36] | 5 | 5 | A | BLDC | ND | 520 | ND | ND | 0.255 | HD | 100 | 0.980 | BLDC | 22.2 | 520 | ND | ND | 0.255 | HD | 50 | 0.980 |
| Penzlin et al 2021 [34] | 4 | 5 | P | BLDC | ND | 260 | 0.494 | 2120 | 0.980 | HD | 50 | 0.520 | BLDC | ND | 260 | 0.494 | 2120 | 0.980 | HD | 50 | 0.520 |
| Tahamipour Zarandi et al 2021 [77] | 2 | 5 | A | BLDC | ND | 90 | ND | ND | ND | HD | 100 | 0.420 | BLDC | ND | 70 | ND | ND | ND | HD | 100 | 0.330 |
| Tanyildizi et al 2021 [52] | 1 | 1 | P | BLDC | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND |
| Cardona et al 2020 [78] | 3 | 5 | R | BLDC | ND | ND | ND | ND | ND | HD | 100 | 0.790 | BLDC | ND | ND | ND | ND | ND | HD | 100 | 0.790 |
| Li et al 2020 [79] | 5 | 5 | R | BLDC | 30 | 260 | 0.992 | 1820 | 0.980 | HD | 50 | 0.980 | BLDC | 24 | 100 | 0.289 | 3740 | 0.355 | HD | 120 | 0.980 |
| Liu et al 2020 [25] | 3 | 5 | P | BLDC | ND | ND | ND | ND | ND | HD | 100 | 0.980 | BLDC | ND | ND | ND | ND | ND | HD | 100 | 0.980 |
| Pirjade et al 2020 [80] | 5 | 1 | A | BLDC | 12 | 80 | ND | 22000 | 0.200 | PG | 234 | 0.227 | BLDC | 12 | 80 | ND | 22000 | 0.200 | PG | 130 | 0.270 |
| Baser et al 2019 [15] | 4 | NP | R | BLDC | ND | 150 | 0.085 | 16600 | 0.300 | NP | NP | NP | BLDC | 24 | 150 | 0.085 | 16600 | 0.300 | NP | NP | NP |
| Chen et al 2019 [81] | 3 | 5 | A | BLDC | ND | ND | ND | ND | ND | HD | 100 | 0.450 | BLDC | ND | ND | ND | ND | ND | HD | 80 | 0.450 |
| Christensen et al 2019 [18] | 3 | 1 | A | BLDC | ND | 100 | 0.269 | 3730 | 0.355 | HD | ND | ND | BLDC | 24 | 100 | 0.269 | 3730 | 0.355 | ND | ND | ND |
| Yang et al 2019 [82] | 5 | 5 | R | BLDC | 24 | 250 | 0.311 | 7970 | 1.150 | PG | 12 | 0.360 | BLDC | 24 | 250 | 0.311 | 7970 | 1.150 | PG | 12 | 0.360 |
| Yildirim et al 2019 [35] | 5 | 5 | A | BLDC | 48 | 280 | 0.940 | 3930 | 0.400 | HD | 100 | 0.240 | BLDC | 48 | 280 | 0.940 | 3930 | 0.400 | HD | 100 | 0.240 |
| Minchala et al 2019 [49] | 1 | 2 | R | BLDC | ND | ND | ND | ND | ND | PG | 196 | ND | ND | ND | ND | ND | ND | ND | PG | ND | ND |
| Zhou et al 2019 [60] | 1 | 1 | A | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND |
| Chen et al 2018 [22] | 5 | 5 | A | BDC | 24 | 150 | 0.177 | 6940 | 0.480 | PG | 113 | 0.460 | BLDC | 24 | 150 | 0.177 | 6940 | 0.480 | PG | 91 | 0.460 |
| Ghezal et al 2018 [23] | 5 | NP | P | BDC | 24 | 150 | 0.177 | 6940 | 0.480 | NP | NP | NP | BDC | 24 | 150 | 0.177 | 6940 | 0.480 | NP | NP | NP |
| He et al 2018 [83] | 4 | 5 | A | BLDC | ND | ND | 0.560 | 2510 | 0.980 | HD | 160 | 1.750 | BLDC | ND | ND | 0.560 | 2510 | 0.980 | HD | 160 | 1.750 |
| Schrade et al 2018 [84] | 4 | 5 | P | BLDC | ND | ND | 0.560 | 3120 | 0.980 | HD | 160 | 1.440 | BLDC | ND | ND | 0.560 | 3120 | 0.980 | HD | 160 | 1.440 |
| Ranaweera et al 2018 [58] | 1 | 1 | A | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND |
| Seo et al 2018 [51] | 1 | 1 | A | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND |
| Chen et al 2017 [16] | 3 | 4 | A | BDC | ND | 150 | 0.177 | 6940 | 0.480 | PG | ND | ND | BLDC | 24 | 150 | 0.177 | 6940 | 0.480 | PG | ND | ND |
| Huang et al 2017 [33] | 3 | 5 | R | BLDC | ND | ND | ND | ND | ND | HD | 160 | 1.200 | BLDC | ND | ND | ND | ND | ND | HD | 100 | 1.200 |
| Vouga et al 2017 [85] | 3 | 5 | A | BLDC | 48 | ND | ND | ND | ND | HD | 100 | 0.650 | BLDC | ND | ND | ND | ND | ND | HD | 100 | 0.650 |
| Zhu et al 2017 [86] | 5 | 5 | R | BLDC | 24 | 250 | 0.311 | 7970 | 1.150 | PG | 12 | 0.360 | BLDC | 24 | 250 | 0.311 | 7970 | 1.150 | PG | 12 | 0.360 |
| Ortlieb et al 2017 [48] | 1 | 1 | R | BLDC | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND |
| Sanngoen et al 2017 [50] | 1 | 1 | A | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND |
| Wu et al 2017 [53] | 1 | 1 | A | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND |
| Tran et al 2016 [87] | 3 | 4 | A | BLDC | ND | 90 | 0.617 | ND | ND | HD | 120 | ND | BLDC | ND | 90 | 0.617 | ND | ND | HD | 120 | ND |
| Cha et al 2015 [88] | 2 | 4 | A | BLDC | 24 | 150 | ND | ND | ND | HD | 233 | ND | BLDC | 24 | 150 | ND | ND | ND | HD | 233 | ND |
| Park et al 2015 [89] | 5 | 5 | R | BLDC | ND | 209 | 0.429 | 7040 | 0.850 | HD | 100 | 0.520 | BLDC | ND | 209 | 0.429 | 7040 | 0.850 | HD | 100 | 0.910 |
| Brackx et al 2014 [32] | 5 | 5 | A | BLDC | 24 | 50 | 0.083 | 5250 | 0.116 | PG | 47 | 0.313 | BLDC | 24 | 50 | 0.083 | 5250 | 0.116 | PG | 47 | 0.310 |
| Fang et al 2014 [90] | 4 | 4 | R | BLDC | ND | 273 | 0.400 | 7750 | ND | PG | 64 | ND | BLDC | ND | 273 | 0.400 | 7750 | ND | PG | 64 | ND |
| Guo et al 2014 [24] | 4 | NP | R | AC motor | ND | 400 | 1.300 | ND | ND | NP | NP | NP | AC motor | ND | 200 | 0.640 | ND | ND | NP | NP | NP |
| Önen et al 2014 [30] | 4 | NP | P | BDC | 24 | 100 | 0.395 | 3000 | ND | NP | NP | NP | BDC | 24 | 100 | 0.395 | 3000 | ND | NP | NP | NP |
| Shi et al 2014 [59] | 1 | 1 | R | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND |
| Zhu et al 2014 [61] | 1 | 1 | P | BLDC | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND |
| Banchadit et al 2012 [47] | 1 | 1 | R | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND |
| Unluhisarcikli et al 2011 [91] | 3 | 4 | R | BLDC | ND | ND | ND | ND | ND | HD | 100 | ND | BLDC | ND | ND | ND | ND | ND | HD | 100 | ND |
| Neuhaus et al 2011 [92] | 4 | 5 | A | BLDC | 50 | ND | 0.450 | 7500 | 1.030 | HD | 160 | 0.520 | BLDC | 50 | ND | 0.450 | 7500 | 1.030 | HD | 160 | 0.520 |
| Farris et al 2011 [56] | 1 | 4 | A | BLDC | ND | ND | ND | ND | ND | PG | 24:1 | ND | BLDC | ND | ND | ND | ND | ND | PG | ND | ND |
| Veneman et al 2007 [44] | 4 | 4 | R | BDC | ND | 567 | 0.870 | 8000 | 1.100 | PG | 64 | ND | BDC | ND | 567 | 0.870 | 8000 | 1.100 | PG | 64 | ND |
| Jamwal et al 2024 [93] | 5 | 5 | Rehabilitation | BLDC | 48 | ND | 48.000 | 40 | 0.850 | PG | 64 | ND | BLDC | 48 | ND | 48.000 | 40 | 0.850 | PG | 64 | ND |