APPENDIX A – TRANSMITTED VARIABLES

Table 1. A description of the relevant meta data transmitted for each capture

| Row/s | Variable | Units | Description |
| --- | --- | --- | --- |
| 1 | time | s | current time (UTC) |
| 3 | ID |  | buoy ID |
| 4 | mode_default |  | default mode for capture rate |
| 5 | mode_mode |  | current mode for capture rate |
| 6 | mode_until | s | length of time in current mode |
| 17 | temperature | degrees | internal temperature of case |
| 20 | battery_voltage | deci-volts | battery voltage |
| 21 | device_uptime | s | uptime of board |
| 22 | device_runtime | s | uptime of software |
| 23 | memory_used | % | memory used |
| 26 | sd_used | % | SD card space used |
| 27 | gps_lat | decimal degrees | latitude at start of capture |
| 28 | gps_lon | decimal degrees | longitude at start of capture |
| 29 | gps_sats |  | number of satellites locked |
| 33 | iridim_signal |  | quality of iridium signal (1 – 5) |

Table 2. A description of the relevant wave data transmitted for each capture

| Row/s | Variable | Units | Description |
| --- | --- | --- | --- |
| 1 – 4 | Hz_max | m | maximum wave height (trough – crest) in a 256 each segment |
| 5 – 8 | Hcm_max | m | maximum height of a crest above mean water level in each 256 s segment |
| 9 – 12 | Htm_max | m | maximum depth of a trough below mean water level in each 256 s segment |
| 13 - 16 | Tz_mean | s | mean wave period in each 256 s segment |
| 17 | Tp | s | peak period from full (non averaged) spectra |
| 18 | Hmo | m | significant wave height from deterministic analysis – 4 x standard deviation of elevation |
| 19 - 73 | psd | m2/Hz | the power spectral density for the 55 wave frequency bins |
| 74-80 | moments |  | spectral moments |
| 90 | qf_mvar | m | quality flag: mean vertical acceleration removed |
| 91 | qf_kist |  | quality flag: Kistler passed (0), Kistler failed (1) |
| 92 | qf_imu |  | quality flag: IMU passed (0), IMU failed (1) |
| 93 | qf_p_kist |  | quality flag: percentage of flagged Kistler samples |
| 94 | qf_p_accel |  | quality flag: percentage of flagged acceleration samples (averaged across 3 axis) |
| 95 | qf_p_gyro |  | quality flag: percentage of flagged gyroscope samples (averaged across 3 axis) |
| 96 | qf_p_mag |  | quality flag: percentage of flagged magnetometer samples (averaged across 3 axis) |
| 98 | power_diff |  | difference between power in the time and frequency domains |
| 101 | Hz_mean | m | deterministic mean wave height |

APPENDIX B – ASPECT OBSERVATIONS

Fig.  18. A map of the location of the ASPeCt observations (small black dots) relative to the inbound deployments (coloured large dots). The lines show the first 24 hrs of the buoy tracks.

Table 3. Details of each ASPeCt observation between buoys during the inbound deployments.

| Buoy ID | Ice Thickness (m) | Floe size (m) | Ice Type (m) | Ice Concentration (%) |
| --- | --- | --- | --- | --- |
| B21 |  |  |  |  |
|  | 0.5 | Pancakes | Pancakes | 100 |
|  | 0.3 | Pancakes | Pancakes | 100 |
|  | 0.3 | Pancakes | Pancakes | 100 |
|  | 0.3 0.3 0.3 0.5 0.3 0.25 0.3 0.5 0.15 | Pancakes Pancakes Pancakes Pancakes Pancakes Pancakes Pancakes Pancakes Small floes (20 – 100) | Pancakes Pancakes Pancakes Pancakes Pancakes Pancakes Pancakes Pancakes Young grey ice (0.1 – 0.15) | 100 100 100 100 100 100 100 80 80 |
| B22 |  |  |  |  |
|  | 0.5 | Pancakes | Pancakes | 100 |
|  | 0.15 | Small floes (20 – 100) | Young grey ice (0.1 – 0.15) | 100 |
|  | 0.5 | Pancakes | Pancakes | 100 |
|  | 0.5 | Cake ice (< 20 m) | Pancakes | 100 |
| B23 |  |  |  |  |
|  | 0.5 | Pancakes | Pancakes | 100 |
|  | 0.5 | Pancakes | Pancakes | 100 |
|  | 0.5 | Pancakes | Pancakes | 100 |
|  | 0.5 | Pancakes | Pancakes | 100 |
|  | 0.5 | Cake ice (< 20 m) | Pancakes | 90 |
|  | 0.4 | Large floes (500 - 2000) | Pancakes | 90 |
| A31 |  |  |  |  |

Fig.  19. A map of the location of the ASPeCt observations (small black dots) relative to the outbound deployments (coloured large dots). The lines show the first 24 hrs of the buoy tracks.

Table 4. Details of each ASPeCt observation between buoys during the outbound deployments.

| Buoy ID | Ice Thickness (m) | Floe size (m) | Ice Type (m) | Ice Concentration (%) |
| --- | --- | --- | --- | --- |
| A32 |  |  |  |  |
|  | 0.6 | Medium floes (100 - 500) | First year (0.3 – 0.7) | 80 |
|  | 0.4 | Medium floes (100 - 500) | First year (0.3 – 0.7) | 80 |
|  | 0.35 | Large floes (500 - 2000) | First year (0.3 – 0.7) | 70 |
|  | 0.15 | Large floes (500 - 2000) | Young grey-white ice (0.15 - 0.3) | 100 |
|  | 0.4 | Vast floes (> 2000) | First year (0.3 – 0.7) | 100 |
|  | 0.4 | Large floes (500 - 2000) | First year (0.3 – 0.7) | 100 |
|  | 0.3 | Large floes (500 - 2000) | First year (0.3 – 0.7) | 100 |
|  | 0.4 | Large floes (500 - 2000) | First year (0.3 – 0.7)) | 90 |
|  | 0.3 | Large floes (500 - 2000) | First year (0.3 – 0.7) | 100 |
|  | 0.35 | Large floes (500 - 2000) | First year (0.3 – 0.7) | 100 |
|  | 0.4 | Large floes (500 - 2000) | First year (0.3 – 0.7) | 100 |
|  | 0.3 | Large floes (500 - 2000) | Young grey-white ice (0.15 - 0.3) | 100 |
|  | 0.3 | Medium floes (100 - 500) | Young grey-white ice (0.15 - 0.3) | 100 |
|  | 0.2 | Medium floes (100 - 500) | Young grey-white ice (0.15 - 0.3) | 100 |
|  | 0.3 | Large floes (500 - 2000) | Young grey-white ice (0.15 - 0.3) | 100 |
|  | 0.2 | Large floes (500 - 2000) | Young grey-white ice (0.15 - 0.3) | 100 |
|  | 0.3 | Medium floes (100 - 500) | Young grey-white ice (0.15 - 0.3) | 100 |
|  | 0.3 | Medium floes (100 - 500) | Young grey-white ice (0.15 - 0.3) | 100 |
|  | 0.4 | Medium floes (100 - 500) | First year (0.3 – 0.7) | 100 |
| A33 |  |  |  |  |
|  | 0.6 | Large floes (500 - 2000) | First year (0.3 – 0.7) | 100 |
|  | 0.25 | Large floes (500 - 2000) | Young grey-white ice (0.15 - 0.3) | 100 |
|  | 0.5 | Vast floes (> 2000) | First year (0.3 – 0.7) | 100 |
|  | 0.25 | Large floes (500 - 2000) | Young grey-white ice (0.15 - 0.3) | 100 |
|  | 0.4 | Large floes (500 - 2000) | First year (0.3 – 0.7) | 100 |
|  | 0.4 | Medium floes (100 - 500) | First year (0.3 – 0.7) | 90 |
|  | 0.5 | Medium floes (100 - 500) | First year (0.3 – 0.7) | 100 |
| A34 |  |  |  |  |
|  | 0.5 | Medium floes (100 - 500) | First year (0.3 – 0.7) | 100 |
|  | 0.25 | Medium floes (100 - 500) | Young grey-white ice (0.15 - 0.3) | 100 |
|  | 0.4 | Small floes (20 - 100) | First year (0.3 – 0.7) | 100 |
|  | 0.6 | Large floes (500 - 2000) | First year (0.3 – 0.7) | 100 |
|  | 0.4 | Medium floes (100 - 500) | First year (0.3 – 0.7) | 100 |
| B24 |  |  |  |  |
|  | 0.3 | Small floes (20 - 100) | Young grey-white ice (0.15 - 0.3) | 100 |
|  | 0.25 | Small floes (20 - 100) | Young grey-white ice (0.15 - 0.3) | 100 |
|  | 0.35 | Small floes (20 - 100) | First year (0.3 – 0.7) | 100 |
|  | 0.4 | Small floes (20 - 100) | First year (0.3 – 0.7) | 100 |
| B25 |  |  |  |  |
|  | 0.75 | Small floes (20 - 100) | First year (0.7 – 1.2) | 100 |
|  | 0.15 | Small floes (20 - 100) | Young grey-white ice (0.15 - 0.3) | 100 |
|  | 0.15 | Large floes (500 - 2000) | Young grey-white ice (0.15 - 0.3) | 100 |
|  | 0.5 | Small floes (20 - 100) | First year (0.3 – 0.7) | 100 |
|  | 0.4 | Small floes (20 - 100) | First year (0.3 – 0.7) | 100 |
| B26 |  |  |  |  |
|  | 0.3 | Small floes (20 - 100) | First year (0.3 – 0.7) | 100 |
|  | 0.3 | Small floes (20 - 100) | First year (0.3 – 0.7) | 100 |
|  | 0.3 | Small floes (20 - 100) | First year (0.3 – 0.7) | 100 |
|  | 0.3 | Small floes (20 - 100) | First year (0.3 – 0.7) | 50 |
| B27 |  |  |  |  |
|  | 0.5 | Small floes (20 - 100) | First year (0.3 – 0.7) | 100 |
|  | 0.5 | Small floes (20 - 100) | First year (0.3 – 0.7) | 100 |
|  | 0.5 | Small floes (20 - 100) | First year (0.3 – 0.7) | 100 |
| B28 |  |  |  |  |
|  | 0.5 | Cake ice (< 20) | First year (0.3 – 0.7) | 100 |
| B29 |  |  |  |  |
|  | 0.3 | Cake ice (< 20) | First year (0.3 – 0.7) | 100 |
|  | 0.3 0.2 0.3 0.3 | Cake ice (< 20) Pancakes Cake ice (< 20) Small floes (20 - 100) | First year (0.3 – 0.7) Pancakes First year (0.3 – 0.7) First year (0.3 – 0.7) | 100 100 90 100 |
| B30 |  |  |  |  |

b

b

APPENDIX C – WIIOS DEPLOYMENTS

Fig.  20. Images of inbound deployments. (a) WIIOS B21 at 03:50 on 21st April 2017 at 69.1715833 S and 171.8200167 E. (b) WIIOS B22 at 10:40 on 21st April 2017 at 69.394453 S and 171.996 E. (c) WIIOS B23 at 16:20 on 21st April 2017 at 69.666 S and 171.9866667 E. (d) WIIOS A31 at 01:20 on 22nd April 2017 at 69.96966667 S and 172.0448333 E. Photo credits: Lettie Roach.

Fig.  21. Images of outbound deployments. (a) WIIOS A32 at 01:00 on 30th May 2017 at 70.5025 S and 176.4193833 W. (b) WIIOS A33 at 12:40 on 1st June 2017 at 69.5055 S and 176.1855833 W. (c) WIIOS A34 at 20:30 on 1st June 2017 at 68.9995 S and 176.1828333 W. (d) WIIOS B24 at 07:00 on 2nd June 2017 at 68.6434167 S and 176.186W. (e) WIIOS B25 at 15:00 on 2nd June 2017 at 68.24175 S and 176.10725 W. (f) WIIOS B26 at 21:00 on 2nd June 2017 at 67.8286833 S and 176.0849333 W. (g) WIIOS B27 at 03:30 on 3rd June 2017 at 67.5772333 S and 175.8371167 W. (h) WIIOS B28 at 08:30 on 3rd June 2017 at 67.33425 S and 175.8421667 W. (i) WIIOS B29 at 11:00 on 3rd June 2017 at 67.14455 S and 175.9256667 W. (j) WIIOS B30 at 12:30 on 3rd June 2017 at 67.0181167 S and 175.9898333 W. Photo credits: Lettie Roach.